M1 Fireline Mapping: Quantified Uncertainty, Version 1.1

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. Version 1.1. Standard day for every mission and fire: 24 h, 06:00 to 06:00 local, on the peak-run day. GOFER not used.

Bottom line

Up to 18 h old. During Carr's 26 July run the newest aircraft perimeter reached 18 h old and the newest ICS-209 12 h, yet station wind allowed every listed drone to fly all 24 hours. A 1 h drone revisit would have held map age near 0.5 h.

Camp is the hard case. Read against gusts, an Alta X could fly about 1 hour in 24 on Camp's standard day (4%), so its map would be most of the day old. A Type 1 fixed wing rated to 30 mph flies 46% of hours: expected map age 1.7 h, about 7,700 ac unmapped at any moment.

The largest unknowns are definitions, not weather. Whether a drone's wind limit applies to sustained wind or gusts, and the unpublished Type 1 limit, move the answer more than any other input.

Who uses it

Incident commander and planning (situation unit, fire behavior analyst): map age decides whether resource orders and evacuation triggers rest on a current line. Division supervisors and lookouts: head and flank position on the active edge, refreshed within the hour.

Drone types serving M1

| Drone | Type | Parameters | Wind limit | M1 role |
| --- | --- | --- | --- | --- |
| L3Harris FVR-90 | NWCG Type 1 FW, VTOL | 12 to 18 h with 8 to 22 lb thermal video; Type 1 class 3,500 to 8,000 ft AGL, 50 mi | Not found (30 and 40 mph run) | Whole-fire perimeter, day and night |
| SuperVolo (NASA Langley) | VTOL FW research | 8 h; 45 to 75 mph; flown as relay in TCL 1 | Not found | Second mapping tier, relay |
| Alta X (Freefly) | NWCG Type 3 rotor | 50 min empty, 22 min at 22 lb; FY22 average sortie 17 min | 22 mph (secondary) | Head and spot detail close in |
| SwitchBlade-Elite | Type 3 band | Up to 50 min, average sortie 18 min; 2 to 2.3 kg | 25 mph (W5) | Close-in edge mapping |

How the uncertainty is quantified

Map error E(t) = G x A(t): area grown since the newest map. With a pass every R hours that succeeds with probability p (the hourly wind gate), E[A] = R (2 - p) / (2p). Sustained wind gives the optimistic p, gusts the conservative p; both are 20 ft winds, so both overstate availability at flight altitude.

| Camp 8 to 9 Nov, R = 1 h | p sustained | p gust | E[A] h | Unseen ac | p gust, Camp nights |
| --- | --- | --- | --- | --- | --- |
| Alta X, 22 mph | 71% | 4% | 0.9 to 23.3 | 4,200 to 106,800 | 62% |
| SwitchBlade-Elite, 25 mph | 75% | 21% | 0.8 to 4.3 | 3,800 to 19,700 | 68% |
| Type 1 if 30 mph | 100% | 46% | 0.5 to 1.7 | 2,300 to 7,700 | 78% |
| Type 1 if 40 mph | 100% | 71% | 0.5 to 0.9 | 2,300 to 4,200 | 94% |

Kinds of uncertainty: p aleatory (weather); wind-limit definition and Type 1 limit epistemic; perimeter area has measurement error (the archive drops from 11,668 ac at 16:34 to 7,016 ac at 19:38 on 25 July, hence the noon rows). E[A] above 24 h or unseen area above 55,000 ac exceeds the standard day and the fire's own growth: read as no useful map. v1.2 needs: Yiming's hourly perimeters as the reference for G; the FVR-90 wind limit; flight-level wind.

Night

On Carr, 10% of the 63 aircraft perimeters were stamped between 20:00 and 06:00, while about 28% of the growth between perimeters fell in those hours (growth spread evenly between perimeters). On Camp, 4% of the 28 aircraft perimeters were stamped between 20:00 and 06:00, while about 46% of the growth between perimeters fell in those hours (growth spread evenly between perimeters). Night wind at the stations allowed an Alta X in 100% of Carr night hours and 62% of Camp's (gusts). Night capability in project records: drones have mapped perimeters at night (ConOps 2026, p.14); the FVR-90 operator states day and night operation (W5); ACERO TCL 1 flights were daytime, within visual line of sight (Wu et al. 2025, p.9); night flight needs a DOI/FAA agreement (NWCG 2025, pp.6-7).

Hour by hour, standard day: Carr, 26 July 06:00 to 27 July 06:00 PDT (Mule Mountain RAWS)

| Hour (N = night) | Aircraft perimeter (age) | ICS-209 (age) | Wind mph sust/gust, RH | Alta X / T1 30 | What the records show | Drone task from this hour |
| --- | --- | --- | --- | --- | --- | --- |
| 26 Jul 06 | 7,016 (11 h) | 6,773 (12 h) | 5/19, 26% | fly/fly |  | Fresh perimeter for the morning plan; decide where the line and resources go |
| 07 | 7,016 (12 h) | 20,000 (1 h) | 7/20, 26% | fly/fly | ICS-209 20,000 ac, 10% contained |  |
| 08 | 7,016 (13 h) | 20,000 (2 h) | 6/20, 27% | fly/fly |  |  |
| 09 | 7,016 (14 h) | 20,000 (3 h) | 6/17, 26% | fly/fly |  |  |
| 10 | 7,016 (15 h) | 20,000 (4 h) | 5/18, 24% | fly/fly |  |  |
| 11 | 7,016 (16 h) | 20,000 (5 h) | 3/14, 22% | fly/fly |  |  |
| 12 | 7,016 (17 h) | 20,000 (6 h) | 3/12, 17% | fly/fly |  | Type 1 holds whole-fire perimeter; Type 3 maps the active east flank |
| 13 | 7,016 (18 h) | 20,000 (7 h) | 3/10, 16% | fly/fly |  |  |
| 14 | 48,237 (1 h) | 20,000 (8 h) | 2/9, 14% | fly/fly | Aircraft perimeter 48,237 ac |  |
| 15 | 48,237 (2 h) | 20,000 (9 h) | 2/9, 12% | fly/fly |  |  |
| 16 | 48,237 (3 h) | 20,000 (10 h) | 1/11, 10% | fly/fly |  |  |
| 17 | 44,484 (0 h) | 20,000 (11 h) | 3/9, 8% | fly/fly | 17:30 run east at 2.5 mph, spotting over 1 mile; 17:34 perimeter 44,484 ac | Track the run hourly: head, spots across the river; push the line to IC and divisions |
| 18 | 44,484 (1 h) | 20,000 (12 h) | 3/21, 7% | fly/fly |  |  |
| 19 | 44,484 (2 h) | 28,763 (0 h) | 2/14, 9% | fly/fly | 19:17 spots east of Sacramento River; 19:20 to 20:00 vortex on radar; 19:45 ICS-209 28,763 ac |  |
| 20 N | 44,484 (3 h) | 28,763 (1 h) | 2/13, 12% | fly/fly | ICS-209 still 28,763 ac (20:15, 21:00) | Night IR perimeter by Type 1 so the morning plan starts from a current line |
| 21 N | 44,484 (4 h) | 28,763 (1 h) | 3/12, 15% | fly/fly |  |  |
| 22 N | 44,484 (5 h) | 28,763 (2 h) | 7/20, 15% | fly/fly |  |  |
| 23 N | 44,484 (6 h) | 28,763 (3 h) | 9/20, 29% | fly/fly |  |  |
| 27 Jul 00 N | 50,050 (1 h) | 28,763 (4 h) | 4/18, 30% | fly/fly | Aircraft perimeter 50,050 ac; spots south of Whiskeytown Lake | Night IR passes on spots and flanks; fresh perimeter before the briefing |
| 01 N | 50,050 (2 h) | 28,763 (5 h) | 6/14, 36% | fly/fly |  |  |
| 02 N | 50,050 (3 h) | 28,763 (6 h) | 5/13, 31% | fly/fly |  |  |
| 03 N | 50,050 (4 h) | 28,763 (7 h) | 6/14, 21% | fly/fly |  |  |
| 04 N | 50,050 (5 h) | 28,763 (8 h) | 2/14, 20% | fly/fly |  |  |
| 05 N | 50,050 (6 h) | 28,763 (9 h) | 4/14, 21% | fly/fly |  |  |

Sources, all in the project folder: Weather Data (RAWS 10-min mean at 20 ft, gust = prior-hour peak); perimeter_supplements time-area records, GOFER rows dropped; Week 6 workbook; UQ proposal v0.5 (W5 = Week 5). Drone tasks and values marked assumed are our proposals, not records.