M2 Spot Fire Detection: Quantified Uncertainty, Version 1.1

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. Version 1.1. Standard day for every mission and fire: 24 h, 06:00 to 06:00 local, on the peak-run day. GOFER not used.

Bottom line

32 to 779 minutes. Expected time for an Alta X searching every 30 min (90% per-pass detection, assumed) to find a new spot fire over Camp's standard day, from the sustained to the gust reading of its limit.

Against Camp's 85 minutes. The first burnovers came 85 min after the first 911 call. Read on gusts, expected detection is 779 min for the Alta X, 145 for the SwitchBlade-Elite and 58 for a Type 1 at 30 mph: only a Type 1 at 30 mph detect inside it.

No spot fire record yet. No drone in the library has an operational spot fire detection record (Week 6 Platform, M2 PARTIAL), so per-pass detection runs as 50%, 80% and 95%.

Who uses it

Lookouts and division supervisors: a spot across the line changes escape routes and safety zones within minutes. Incident commander and operations: detection time decides whether initial attack resources reach a spot before it becomes a new front.

Drone types serving M2

| Drone | Type | Parameters | Wind limit | M2 role |
| --- | --- | --- | --- | --- |
| Type 1 and 2 FW (FVR-90, ScanEagle, Aerosonde class) | NWCG Type 1 / 2 | Type 1: 6 to 14 h, mid-wave IR; Type 2: 1 to 6 h, long-wave IR; above all other incident aircraft | Not found | Wide-area IR search, day and night |
| Alta X / SwitchBlade-Elite | NWCG Type 3 | 17 to 18 min average sorties; 5 mi range | 22 / 25 mph | Initial attack size-up; confirm and geolocate hot spots |
| DJI Mavic 2 Enterprise Advanced (sensor reference) | Type 4 band, research | 640 x 512 thermal, 8 to 14 µm, plus 4K RGB; 60 m AGL in FLAME 2; thermal -40 to 550 °C low gain | Not in library | Fused RGB-IR flame detection benchmark |

How the uncertainty is quantified

Time to detect: a pass every R min that flies with p (wind gate) and sees the spot with Pd finds it on pass G ~ Geometric(q), q = p x Pd: E[T_detect] = R/2 + R (1 - q)/q. Smallest detectable spot A_min = (k x GSD)^2, GSD = 2h tan(HFOV/2)/N. With an assumed 40° HFOV and k = 3 pixels, a 640-pixel array gives GSD 6.8 cm at 60 m AGL (A_min 0.04 m²) and 1.21 m at 3,500 ft (A_min 13 m²).

| Camp 8 to 9 Nov, R = 30 min (assumed) | Pd 50%: min | Pd 80%: min | Pd 95%: min |
| --- | --- | --- | --- |
| Alta X, 22 mph | 70 to 1414 | 38 to 878 | 30 to 737 |
| SwitchBlade-Elite, 25 mph | 65 to 273 | 35 to 165 | 27 to 137 |
| Type 1 if 30 mph | 45 to 116 | 22 to 67 | 17 to 54 |
| Type 1 if 40 mph | 45 to 70 | 22 to 38 | 17 to 30 |

Kinds of uncertainty: p aleatory (gusts); Pd epistemic until flight-tested in smoke and at night; HFOV and k assumed. v1.2 needs: Pd and false alarm rate by smoke and night class; search area per fire (Week 6 Q2); the Type 1 wind limit.

Night and heat sensors

Thermal sees flame and hot ground through smoke and darkness where RGB and lookouts cannot; the Mavic 2 EA range saturates at 550 °C, so it locates flame without measuring intensity. Smoke detection suits early detection and flame detection larger fires (Keerthinathan et al. 2023, Week 6 M2 Q5). On Camp, 4% of the 28 aircraft perimeters were stamped between 20:00 and 06:00, while about 46% of the growth between perimeters fell in those hours (growth spread evenly between perimeters). Camp night gusts allowed an Alta X in 62% of night hours and a Type 1 at 30 mph in 78%. Night capability in project records: drones have mapped perimeters at night (ConOps 2026, p.14); the FVR-90 operator states day and night operation (W5); ACERO TCL 1 flights were daytime, within visual line of sight (Wu et al. 2025, p.9); night flight needs a DOI/FAA agreement (NWCG 2025, pp.6-7).

Hour by hour, standard day: Camp, 8 November 06:00 to 9 November 06:00 PST (Jarbo Gap RAWS, on the fire)

| Hour (N = night) | Aircraft perimeter (age) | ICS-209 (age) | Wind mph sust/gust, RH | Alta X / T1 30 | What the records show | Drone task from this hour |
| --- | --- | --- | --- | --- | --- | --- |
| 8 Nov 06 | none | none | 18/40, 23% | no/no | 06:25 first 911 call; 06:44 first engine, about 35 mph wind; 06:45 second ignition | Initial attack: first look and size-up from the engine or station drone |
| 07 | none | none | 14/28, 21% | no/fly | 07:10 engine estimate 200 to 300 ac; 07:37 Concow order; 07:50 first burnovers; 07:52 air attack 1,000 ac | Search ahead of the head for spots toward Concow and Paradise; geolocate each for engines |
| 08 | none | none | 6/25, 18% | no/fly | 08:01 'fire is now in Paradise', 2,000 ac; 08:29 ICS-209 discovery time |  |
| 09 | 54,586 (0 h) | none | 14/21, 17% | fly/fly | 09:03 Paradise and Magalia under orders; 09:54 first aircraft perimeter 54,586 ac | Wide-area IR search downwind; confirm spots that threaten evacuation routes |
| 10 | 54,586 (1 h) | none | 18/30, 16% | no/fly | 10:45 Landsat 8 image (no area given) |  |
| 11 | 54,586 (2 h) | none | 14/29, 14% | no/fly |  |  |
| 12 | 54,586 (3 h) | none | 16/33, 13% | no/no |  |  |
| 13 | 54,586 (4 h) | none | 12/32, 12% | no/no |  |  |
| 14 | 54,586 (5 h) | none | 15/24, 11% | no/fly |  |  |
| 15 | 54,586 (6 h) | none | 10/28, 12% | no/fly |  |  |
| 16 | 70,454 (1 h) | none | 8/25, 12% | no/fly | Aircraft perimeter 70,454 ac; satellite 81,810 ac |  |
| 17 | 70,454 (2 h) | none | 13/23, 11% | no/fly |  |  |
| 18 | 70,454 (3 h) | 16,000 (0 h) | 15/27, 12% | no/fly | Green Sheet: first 12 h ran 15 mi, over 55,000 ac, spotting up to a mile ahead; 18:30 first ICS-209 16,000 ac | Night IR search for spots outside the line; send positions to night operations |
| 19 | 70,454 (4 h) | 18,000 (1 h) | 18/30, 12% | no/fly | 19:20 ICS-209 18,000 ac; long-range spotting reported |  |
| 20 N | 70,454 (5 h) | 18,000 (2 h) | 19/34, 12% | no/no |  |  |
| 21 N | 70,454 (6 h) | 18,000 (3 h) | 22/36, 13% | no/no |  |  |
| 22 N | 70,454 (7 h) | 18,000 (4 h) | 22/39, 14% | no/no |  |  |
| 23 N | 70,454 (8 h) | 18,000 (5 h) | 27/43, 14% | no/no |  |  |
| 9 Nov 00 N | 70,454 (9 h) | 18,000 (6 h) | 25/48, 14% | no/no |  | Night IR search; list confirmed spots for the morning briefing |
| 01 N | 70,454 (10 h) | 18,000 (7 h) | 28/45, 15% | no/no |  |  |
| 02 N | 70,454 (11 h) | 18,000 (8 h) | 27/48, 15% | no/no |  |  |
| 03 N | 70,454 (12 h) | 18,000 (9 h) | 28/41, 16% | no/no |  |  |
| 04 N | 70,454 (13 h) | 18,000 (10 h) | 28/47, 17% | no/no | Satellite 92,880 ac |  |
| 05 N | 70,454 (14 h) | 18,000 (11 h) | 29/45, 17% | no/no |  |  |

Sources, all in the project folder: Weather Data (RAWS 10-min mean at 20 ft, gust = prior-hour peak); perimeter_supplements time-area records, GOFER rows dropped; Week 6 workbook; UQ proposal v0.5 (W5 = Week 5). Drone tasks and values marked assumed are our proposals, not records.