M3 Firefighter Safety: Quantified Uncertainty, Version 1.1

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. Version 1.1. Standard day for every mission and fire: 24 h, 06:00 to 06:00 local, on the peak-run day. GOFER not used.

Bottom line

P(M < 0), the chance the fire reaches the crew before they are out, for a 45-minute escape at Camp's first burnovers: 10% with an Alta X if gusts are ignored, 91% if not, 28% with a Type 1 rated to 30 mph.

Carr: flyable, still at risk. Mule Mountain gusts stayed at or below 21 mph all standard day, so every listed drone could fly; P(M < 0) for a 45-minute escape is still 21%, half from a missed pass, half from relay and egress.

Escape time is the missing measurement. Crew positions and escape routes at the burnovers are not in project records, so results are shown for 30, 45 and 60 minutes.

Who uses it

Crew level, lookouts and crew bosses: their number; a warning must arrive with more time than the escape takes. Division supervisors and the safety officer: trigger points, escape routes and safety zones. Incident commander: whether a crew is committed in the conditions.

Drone types serving M3

| Drone | Type | Parameters | Wind limit | M3 role |
| --- | --- | --- | --- | --- |
| FVR-90 | NWCG Type 1 FW | 12 to 18 h; thermal video; flew as airborne relay in TCL 1 | Not found | Overwatch of crews and escape routes; relay |
| SuperVolo | VTOL FW research | 8 h; relay at 1,200 to 1,500 ft MSL | Not found | Alternate relay behind terrain |
| Alta X | NWCG Type 3 rotor | 17 min average sortie; under 2,000 ft AGL | 22 mph (secondary) | Check escape routes, safety zones, landing zones |
| DJI Matrice 200 v2 (research) | Type 3 / 4 band | Sonic anemometer profiles near the fire front | Not in library | Local wind for trigger points |

How the uncertainty is quantified

M = T_fire - (T_detect + T_notify + T_egress), headline P(M < 0). T_detect: pass every 15 min (assumed), flies with wind gate p, sees the hazard with Pd 0.9 (assumed). T_notify: retries every 2 min (assumed), each delivered with s = 52%, the TCL 1 telemetry sharing rate with four ground nodes (Wu et al. 2025, pp.9-11), a stand-in for alert delivery. T_fire: 85 min at Camp (06:25 call to 07:50 burnovers), 62 min at Carr (18:38 aerial warning to 19:40 mayday). Monte Carlo, 20,000 draws per cell.

| P(M < 0) by escape time | 30 min | 45 min | 60 min |
| --- | --- | --- | --- |
| Camp first burnovers, Alta X, sustained | 3% | 10% | 27% |
| Camp first burnovers, Alta X, gusts | 88% | 91% | 95% |
| Camp first burnovers, Type 1 if 30 mph, gusts | 17% | 28% | 49% |
| Carr 26 July, any listed drone, gusts | 2% | 21% | 100% |

Kinds of uncertainty: p aleatory (gusts); Pd, pass interval and retry assumed; s measured once in TCL 1; T_egress epistemic and the largest. v1.2 needs: crew positions and escape routes at the Camp and Carr burnovers; radio and dispatch logs for real T_notify; a measured alert delivery rate.

Night

On Camp, 4% of the 28 aircraft perimeters were stamped between 20:00 and 06:00, while about 46% of the growth between perimeters fell in those hours (growth spread evenly between perimeters). Camp's burnovers came in daylight, but night wind still grounded an Alta X in 38% of Camp night hours by gusts. Crews working at night lose the lookout's view, so a thermal drone overhead is the only continuous watch of the edge. Night capability in project records: drones have mapped perimeters at night (ConOps 2026, p.14); the FVR-90 operator states day and night operation (W5); ACERO TCL 1 flights were daytime, within visual line of sight (Wu et al. 2025, p.9); night flight needs a DOI/FAA agreement (NWCG 2025, pp.6-7).

Hour by hour, standard day: Camp, 8 November 06:00 to 9 November 06:00 PST (Jarbo Gap RAWS)

| Hour (N = night) | Aircraft perimeter (age) | ICS-209 (age) | Wind mph sust/gust, RH | Alta X / T1 30 | What the records show | Drone task from this hour |
| --- | --- | --- | --- | --- | --- | --- |
| 8 Nov 06 | none | none | 18/40, 23% | no/no | 06:44 first engine; about 35 mph wind | Watch crews and engines on Concow Rd; check escape routes ahead of the head |
| 07 | none | none | 14/28, 21% | no/fly | 07:37 Concow order; 07:50 first burnovers (Hoffman Rd, Concow Rd) |  |
| 08 | none | none | 6/25, 18% | no/fly | 08:01 fire in Paradise; burnovers continue | Overwatch of crews in Paradise; mark safety zones and blocked routes; relay warnings |
| 09 | 54,586 (0 h) | none | 14/21, 17% | fly/fly | 09:03 Paradise and Magalia under orders; 09:54 first aircraft perimeter |  |
| 10 | 54,586 (1 h) | none | 18/30, 16% | no/fly |  |  |
| 11 | 54,586 (2 h) | none | 14/29, 14% | no/fly |  |  |
| 12 | 54,586 (3 h) | none | 16/33, 13% | no/no |  |  |
| 13 | 54,586 (4 h) | none | 12/32, 12% | no/no |  |  |
| 14 | 54,586 (5 h) | none | 15/24, 11% | no/fly | 14:15 firing operation, Rattlesnake Flats Rd; 14:45 wind shift, fire crosses road around crew | Before and during firing: wind and fire position over Rattlesnake Flats; relay when radios saturate |
| 15 | 54,586 (6 h) | none | 10/28, 12% | no/fly | 15:00 emergency call unanswered; 15:15 burnover |  |
| 16 | 70,454 (1 h) | none | 8/25, 12% | no/fly | Aircraft perimeter 70,454 ac | Reassess safety zones and helicopter landing zones for the night |
| 17 | 70,454 (2 h) | none | 13/23, 11% | no/fly |  |  |
| 18 | 70,454 (3 h) | 16,000 (0 h) | 15/27, 12% | no/fly | 18:30 first ICS-209: 16,000 ac |  |
| 19 | 70,454 (4 h) | 18,000 (1 h) | 18/30, 12% | no/fly |  |  |
| 20 N | 70,454 (5 h) | 18,000 (2 h) | 19/34, 12% | no/no |  | Night overwatch of crews on the line; thermal view replaces the lookout |
| 21 N | 70,454 (6 h) | 18,000 (3 h) | 22/36, 13% | no/no |  |  |
| 22 N | 70,454 (7 h) | 18,000 (4 h) | 22/39, 14% | no/no |  |  |
| 23 N | 70,454 (8 h) | 18,000 (5 h) | 27/43, 14% | no/no |  |  |
| 9 Nov 00 N | 70,454 (9 h) | 18,000 (6 h) | 25/48, 14% | no/no |  | Night overwatch; confirm escape routes are open for the morning shift |
| 01 N | 70,454 (10 h) | 18,000 (7 h) | 28/45, 15% | no/no |  |  |
| 02 N | 70,454 (11 h) | 18,000 (8 h) | 27/48, 15% | no/no |  |  |
| 03 N | 70,454 (12 h) | 18,000 (9 h) | 28/41, 16% | no/no |  |  |
| 04 N | 70,454 (13 h) | 18,000 (10 h) | 28/47, 17% | no/no |  |  |
| 05 N | 70,454 (14 h) | 18,000 (11 h) | 29/45, 17% | no/no |  |  |

Sources, all in the project folder: Weather Data (RAWS 10-min mean at 20 ft, gust = prior-hour peak); perimeter_supplements time-area records, GOFER rows dropped; Week 6 workbook; UQ proposal v0.5 (W5 = Week 5). Drone tasks and values marked assumed are our proposals, not records.