M5 Logistics Support: Quantified Uncertainty, Version 1.1

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. Version 1.1. Standard day for every mission and fire: 24 h, 06:00 to 06:00 local, on the peak-run day. GOFER not used.

Bottom line

7.1 km becomes 0.9 km, then zero. Loaded radius of an Alta X with 22 lb (10 kg, about 10 L of water): still air, Camp's 29 mph sustained headwind, and its 48 mph gusts (above the assumed 50 km/h cruise; its 22 mph limit grounds it first).

P(on time), share of standard-day hours: Camp 4% to 71%, LNU 62% to 79%, Pioneer 96% to 100%, for a 3 km delivery inside 30 minutes (gust to sustained reading).

Small drones carry litres, crews need loads. About 3.1 deliveries per hour, 31 kg/h per Alta X; heavy-lift aircraft in development (Elroy Chaparral, 500 lb over 300 nm; Piasecki KARGO II, 1,500 lb) are the M5 class.

Who uses it

Crew bosses and division supervisors: whether water, hose, pumps and food arrive when needed on remote lines (Pioneer) or when roads are jammed by evacuation (Camp). Logistics section chief and incident commander: how many aircraft to order, which crews to supply by air, and which helicopter landing zones stay usable.

Aircraft serving M5

| Aircraft | Status | Payload and endurance | Wind limit | M5 role |
| --- | --- | --- | --- | --- |
| Alta X (Freefly) | In service, flown with ACERO | 35 lb max; 50 min empty, 35 min at 8 lb, 22 min at 22 lb; 60 mph top speed | 22 mph (secondary) | Light resupply under 10 kg; landing zone checks |
| Type 3 / 4 agency small UAS | In service | Light loads; Type 3 range 5 mi, Type 4 under 2 mi | Not found | Radios, batteries, first aid |
| Elroy Air Chaparral | In development (W5) | 500 lb over 300 nm | Not found | Crew resupply from staging |
| Piasecki KARGO II | In development (W5) | 1,500 lb | Not found | Water, hose and pumps in bulk |

How the uncertainty is quantified

P(on time) = p x P(complete | fly) x P(cycle <= window). Loaded radius into headwind w at airspeed V: r = t_usable / (1/(V - w) + 1/(V + w)). Cycle = round trip + hold + turnaround (Week 6 M5 Q4). Endurance 22 min at 22 lb (W5); reserve 5 min, V = 50 km/h, hold 2 min, turnaround 10 min, crew at 3 km all assumed. P(complete | fly) = 1 until a delivery success rate is measured. P(on time) is the share of critical-window hours meeting both the wind limit and the window.

| Alta X, 10 kg | Radius km at peak sust. / gust | Cycle min at peak sust. / gust | P(on time), standard-day hours | p gust, nights |
| --- | --- | --- | --- | --- |
| Camp | 0.9 / 0.0 | 68 / none | 4% to 71% | 62% |
| LNU | 0.9 / 0.0 | 68 / none | 62% to 79% | 92% |
| Pioneer | 6.5 / 2.5 | 20 / 32 | 96% to 100% | 99% |
| Carr | 6.5 / 3.8 | 20 / 25 | 100% to 100% | 100% |

Kinds of uncertainty: p and w aleatory (station wind); airspeed, turnaround and success rate assumed; crew demand is not in project records, so fleet size is not computed. v1.2 needs: crew demand per shift; measured Alta X loaded cruise speed and wind limit; distances from staging to crews on Pioneer and Camp.

Night

On Pioneer, 21% of the 89 aircraft perimeters were stamped between 20:00 and 06:00, while about 40% of the growth between perimeters fell in those hours (growth spread evenly between perimeters). Pioneer night gusts allowed an Alta X in 99% of night hours and LNU's in 92%, so wind is rarely what stops a night delivery; authorisation is. Night capability in project records: drones have mapped perimeters at night (ConOps 2026, p.14); the FVR-90 operator states day and night operation (W5); ACERO TCL 1 flights were daytime, within visual line of sight (Wu et al. 2025, p.9); night flight needs a DOI/FAA agreement (NWCG 2025, pp.6-7).

Hour by hour, standard day: Pioneer, 29 August 06:00 to 30 August 06:00 MDT (Bearskin Creek RAWS)

| Hour (N = night) | Aircraft perimeter (age) | ICS-209 (age) | Wind mph sust/gust, RH | Alta X / T1 30 | What the records show | Drone task from this hour |
| --- | --- | --- | --- | --- | --- | --- |
| 29 Aug 06 | 111,658 (13 h) | 109,594 (12 h) | 4/6, 46% | fly/fly | 00:00 NIROPS boundary labelled (Kingsmill et al.); about 29,000 ac of growth on 29 Aug | Resupply crews on remote lines (water, batteries, radios) before the afternoon |
| 07 | 111,658 (14 h) | 109,594 (13 h) | 4/7, 47% | fly/fly |  |  |
| 08 | 111,658 (15 h) | 109,594 (14 h) | 5/8, 43% | fly/fly |  |  |
| 09 | 111,658 (16 h) | 109,594 (15 h) | 4/8, 38% | fly/fly |  |  |
| 10 | 111,658 (17 h) | 109,594 (16 h) | 3/8, 36% | fly/fly |  |  |
| 11 | 111,658 (18 h) | 109,594 (17 h) | 2/5, 28% | fly/fly |  |  |
| 12 | 111,658 (19 h) | 109,594 (18 h) | 1/5, 25% | fly/fly |  | Scout helicopter landing zones and drop points ahead of the run |
| 13 | 111,658 (20 h) | 109,594 (19 h) | 3/11, 21% | fly/fly |  |  |
| 14 | 111,658 (21 h) | 109,594 (20 h) | 3/11, 23% | fly/fly |  |  |
| 15 | 111,658 (22 h) | 109,594 (21 h) | 4/13, 17% | fly/fly |  |  |
| 16 | 111,658 (23 h) | 109,594 (22 h) | 4/17, 17% | fly/fly |  | Hold resupply; keep landing zones and safety zones current as the run builds |
| 17 | 111,658 (24 h) | 109,594 (23 h) | 4/18, 17% | fly/fly |  |  |
| 18 | 140,777 (1 h) | 109,594 (24 h) | 7/18, 18% | fly/fly | 18:16 aircraft perimeter 140,778 ac; 19:00 ICS-209 111,604 ac, 58% contained |  |
| 19 | 140,777 (2 h) | 111,604 (1 h) | 9/25, 16% | no/fly |  |  |
| 20 N | 140,777 (3 h) | 111,604 (2 h) | 5/19, 18% | fly/fly |  |  |
| 21 N | 140,777 (4 h) | 111,604 (3 h) | 3/9, 18% | fly/fly |  | Night resupply to crews holding line, if authorised |
| 22 N | 140,777 (5 h) | 111,604 (4 h) | 1/5, 18% | fly/fly |  |  |
| 23 N | 140,777 (6 h) | 111,604 (5 h) | 1/2, 17% | fly/fly |  |  |
| 30 Aug 00 N | 140,777 (7 h) | 111,604 (6 h) | 1/3, 20% | fly/fly | 00:16 NIROPS boundary labelled (Kingsmill et al.) | Night: stage loads and check landing zones for first light |
| 01 N | 140,777 (8 h) | 111,604 (7 h) | 2/4, 19% | fly/fly |  |  |
| 02 N | 140,777 (9 h) | 111,604 (8 h) | 3/4, 25% | fly/fly |  |  |
| 03 N | 140,777 (10 h) | 111,604 (9 h) | 3/5, 25% | fly/fly |  |  |
| 04 N | 140,777 (11 h) | 111,604 (10 h) | 3/5, 26% | fly/fly |  |  |
| 05 N | 140,777 (12 h) | 111,604 (11 h) | 3/5, 28% | fly/fly |  |  |

Sources, all in the project folder: Weather Data (RAWS 10-min mean at 20 ft, gust = prior-hour peak); perimeter_supplements time-area records, GOFER rows dropped; Week 6 workbook; UQ proposal v0.5 (W5 = Week 5). Drone tasks and values marked assumed are our proposals, not records.