M1 Fireline Mapping: Quantified Uncertainty, Version 1.0

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. First version of the M1 uncertainty calculation; builds on UQ proposal v0.5.

Bottom line

12.5 h versus 0.5 h. Expected age of the newest Camp fireline map for an Alta X flying a 1 h revisit on the first day: 0.5 h if the 22 mph limit is read against sustained wind, 12.5 h if it is read against gusts (flyable in 8% of hours at Jarbo Gap). The gap between the two readings is larger than the drone's benefit.

Most of the first day unseen. Read against gusts, the Alta X flies in about 1 of 13 hours, so its newest map would typically be most of the day old; the steady-state 12.5 h is an upper bound longer than the window itself. At Camp's average first-day growth (more than 55,000 ac in 12 h, about 4,583 ac/h) a Type 1 fixed wing rated to 30 mph leaves about 3,700 ac unmapped at any moment.

Gusts ground the drones, and the Type 1 limit is the largest unknown. At Jarbo Gap, on the Camp fire, sustained wind peaked at 18 mph and gusts at 40; other Camp stations reached 25 to 29 mph sustained and 59 mph gusts, so the Alta X gust gate runs 0% to 100% by station. No FVR-90 class limit is published; Camp flyable hours move from 77% to 100% between a 30 and a 40 mph limit.

Who uses it

Incident commander and planning (situation unit, fire behavior analyst): map age decides whether resource orders and evacuation triggers rest on a current line. Division supervisors and lookouts: head and flank position on the active edge, refreshed within the hour.

Drone types serving M1 and their parameters

| Drone | Type | Parameters | Wind limit | M1 role |
| --- | --- | --- | --- | --- |
| L3Harris FVR-90 | NWCG Type 1 FW, VTOL | 12 to 18 h with 8 to 22 lb thermal video; Type 1 class: 3,500 to 8,000 ft AGL, 50 mi range | Not found (30 and 40 mph run) | Whole-fire perimeter every pass |
| SuperVolo (NASA Langley) | VTOL FW research | 8 h; 45 to 75 mph cruise; flown as relay in TCL 1 | Not found | Second mapping tier, relay |
| Alta X (Freefly) | NWCG Type 3 rotor | 50 min empty, 22 min at 22 lb; FY22 average sortie 17 min; 2,000 ft AGL and below | 22 mph (secondary) | Head and spot detail close in |
| SwitchBlade-Elite | Type 3 band | Up to 50 min, average sortie 18 min; 2 to 2.3 kg payload | 25 mph (W5) | Close-in edge mapping |

Live weather on the fires and what it means for tracking the fireline

| Fire | Critical window | Station | Sust. max mph | Gust max mph | Min RH % | Min 10-h fuel moist. % | Wind from |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Camp | 8 Nov 2018, 06:00 to 18:00 PST | JBGC1 Jarbo Gap Raws | 18 | 40 | 11 | 5 | NNE to NE (14 to 42°) |
| Carr | 26 Jul 2018, 12:00 to 23:00 PDT | MMOC1 Shasta - Mule Mountain | 9 | 21 | 7 | 2 | variable, W to NW by evening |
| Pioneer | 29 Aug 2016, 08:00 to 23:00 MDT | BCFI1 Bearskin Creek | 9 | 25 | 16 | n/a | E in the morning, SW to W after 15:00 |
| LNU | 18 Aug 12:00 to 19 Aug 2020 12:00 PDT | ATLC1 Atlas Peak Raws | 29 | 41 | 8 | 4 | SW to WSW (213 to 255°) to 20:00, then WNW |

Wind direction places the head and sets where the drone flies first. At Camp the NE wind at Jarbo Gap drove the head southwest toward Paradise (air attack, 08:01); a 1 h revisit maps that head every pass and the flanks every other pass. At LNU, SW to WSW wind at Atlas Peak rose to 29 mph sustained, 41 mph gusts by 18:00 on 18 August, hours before the overnight merge. Pioneer's 29 August run happened with surface gusts of 25 mph or less: it had pyroconvective updrafts (v0.5), so a surface wind gate alone would wrongly clear drones to fly near it.

Tracking upslope runs and fuels. Slope from USGS 3DEP marks the flanks where a run will accelerate upslope; those flanks get the next pass after the head. Fuels were dry (10-h moisture 2 to 5%, table above). Next: LANDFIRE fuel loading against drone-derived fuel maps, burn history from Landsat in Google Earth Engine, and the burning index from the RAWS records.

How the uncertainty is quantified

Fireline map error at time t: E(t) = G x A(t), the area grown since the newest map, where G is the growth rate (ac/h) and A(t) the information age (h). With a pass due every R hours that succeeds with probability p, the gaps between good maps are R x Geometric(p), so the expected age is E[A] = R (2 - p) / (2p). p is the hourly wind gate, the share of hours the station wind is at or under the drone's limit: sustained wind gives the optimistic bound, gusts the conservative one. Both are 20 ft winds; flight-level wind is stronger, so both overstate availability.

| Camp (Jarbo Gap), R = 1 h | p sustained | p gust | E[A] h | Unseen ac | p gust, whole Camp period |
| --- | --- | --- | --- | --- | --- |
| Alta X, 22 mph | 100% | 8% | 0.5 to 12.5 | 2,300 to 57,200 | 68% |
| SwitchBlade-Elite, 25 mph | 100% | 38% | 0.5 to 2.1 | 2,300 to 9,600 | 74% |
| Type 1 FW if 30 mph | 100% | 77% | 0.5 to 0.8 | 2,300 to 3,700 | 82% |
| Type 1 FW if 40 mph | 100% | 100% | 0.5 to 0.5 | 2,300 to 2,300 | 96% |

E[A] above 12 h exceeds the 13 h window and unseen area above 55,000 ac exceeds the fire's own 12 h growth: read those cells as 'no useful map'. Growth was front-loaded, so a constant G understates the early hours.

Kinds of uncertainty. p: aleatory (weather), measured hourly. Type 1 limit and gust versus sustained reading: epistemic, fixed by asking Overwatch Aero or NASA. Assumes passes independent hour to hour; gusty hours cluster, which lengthens the worst gaps.

v1.1 needs: Yiming's hourly perimeters as the reference for G (replaces GOFER); the FVR-90 wind limit; flight-level wind (HRRR); a plume term for Pioneer-type runs; perimeter length and swath to turn R into a fleet size (Week 6 Q1).

Sources: Weather Data folder via weather_evaluation.py (RAWS 10-min mean at 20 ft; gust = prior-hour peak); Week 6 workbook; UQ proposal v0.5 (W5 = Week 5 Platform tab). Assumed values are labelled.