M2 Spot Fire Detection: Quantified Uncertainty, Version 1.0

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. First version of the M2 uncertainty calculation; builds on UQ proposal v0.5.

Bottom line

18 to 418 minutes. Expected time to detect a new spot fire at Camp on the first day for an Alta X searching every 30 min with a 90% per-pass detection probability, depending on whether its 22 mph limit is read against sustained wind or gusts.

Against Camp's 85 minutes. Only the Alta X read against gusts (418 min) misses it; the SwitchBlade-Elite (72 min) and a Type 1 at 30 mph (28 min) detect inside it at Pd 0.9. The 85 minutes runs from the first 911 call to the first burnovers, so it is a comparison, not a deadline.

Sensor probability is the second unknown. No drone in the library has an operational spot fire detection record (Week 6 Platform, M2 PARTIAL), so per-pass detection Pd runs as 50%, 80% and 95%.

Who uses it

Lookouts and division supervisors: a spot fire across the line changes escape routes and safety zones within minutes, so the alert must reach them first. Incident commander and operations: detection time sets whether initial attack resources are ordered before the spot becomes a new front.

Drone types serving M2 and their parameters

| Drone | Type | Parameters | Wind limit | M2 role |
| --- | --- | --- | --- | --- |
| Type 1 and 2 FW (FVR-90, ScanEagle, Aerosonde class) | NWCG Type 1 / 2 | Type 1: 6 to 14 h, mid-wave IR; Type 2: 1 to 6 h, long-wave IR; above all other incident aircraft | Not found | Wide-area IR search |
| Alta X / SwitchBlade-Elite | NWCG Type 3 | 17 to 18 min average sorties; 5 mi range | 22 / 25 mph | Confirm and geolocate flagged hot spots, cut false alarms |
| DJI Mavic 2 Enterprise Advanced (sensor reference) | Type 4 band, research | 640 x 512 uncooled thermal, 8 to 14 µm, plus 4K RGB; flown at 60 m AGL for FLAME 2 | Not in library | Fused RGB-IR flame detection benchmark |

Live weather on the fires and what it means for spotting

| Fire | Critical window | Station | Sust. max mph | Gust max mph | Min RH % | Min 10-h fuel moist. % | Wind from |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Camp | 8 Nov 2018, 06:00 to 18:00 PST | JBGC1 Jarbo Gap Raws | 18 | 40 | 11 | 5 | NNE to NE (14 to 42°) |
| Pioneer | 29 Aug 2016, 08:00 to 23:00 MDT | BCFI1 Bearskin Creek | 9 | 25 | 16 | n/a | E in the morning, SW to W after 15:00 |
| LNU | 18 Aug 12:00 to 19 Aug 2020 12:00 PDT | ATLC1 Atlas Peak Raws | 29 | 41 | 8 | 4 | SW to WSW (213 to 255°) to 20:00, then WNW |
| Carr | 26 Jul 2018, 12:00 to 23:00 PDT | MMOC1 Shasta - Mule Mountain | 9 | 21 | 7 | 2 | variable, W to NW by evening |

Low humidity and gusts are what make spot fires, and they are also what ground the small drones. At Jarbo Gap relative humidity fell from 23% at 06:00 to 11% by 14:00 with gusts of 21 to 40 mph; at LNU Hawkeye RAWS held 6 to 13% overnight. The search must therefore be planned for the hours the Type 3 tier cannot fly, which points M2 to the Type 1 tier if its wind limit is 30 mph or higher.

What heat sensors tell us. Thermal sees flame and hot ground through smoke when visibility defeats RGB and lookouts; fused RGB-IR is the FLAME 2 benchmark. The Mavic 2 EA thermal range is -40 to 550 °C in low gain, so it locates flame but saturates before measuring intensity. Smoke detection suits early detection and flame detection suits larger fires (Keerthinathan et al. 2023, Week 6 M2 Q5).

How the uncertainty is quantified

Time to detect: with a search pass every R minutes, a per-pass chance the drone flies p (wind gate) and a per-pass chance the sensor sees the spot Pd, the spot is seen on pass G ~ Geometric(q), q = p x Pd. Expected detection time E[T_detect] = R/2 + R (1 - q)/q. Smallest detectable spot: A_min = (k x GSD)^2, GSD = 2 h tan(HFOV/2) / N for an N-pixel array at height h. Example (assumed 40° HFOV, k = 3 pixels): a 640-pixel array at 60 m AGL gives GSD 6.8 cm and A_min about 0.04 m²; at 3,500 ft AGL (Type 1 floor) GSD 1.21 m and A_min about 13 m².

| Camp day 1, R = 30 min | Pd 50%: minutes | Pd 80%: minutes | Pd 95%: minutes |
| --- | --- | --- | --- |
| Alta X, 22 mph | 45 to 764 | 22 to 472 | 17 to 395 |
| SwitchBlade-Elite, 25 mph | 45 to 141 | 22 to 82 | 17 to 67 |
| Type 1 FW if 30 mph | 45 to 63 | 22 to 34 | 17 to 26 |
| Type 1 FW if 40 mph | 45 to 45 | 22 to 22 | 17 to 17 |

Kinds of uncertainty. p: aleatory, from the station gusts. Pd: epistemic until flight-tested in smoke (Week 6 M2 Q1). HFOV and k: assumed, to be replaced by the chosen payload's specification. Ranges in the table run from the sustained-wind to the gust reading.

v1.1 needs: Pd and false alarm rate from flight tests by smoke class; search area per fire (Week 6 Q2) to set R from fleet size; the Type 1 wind limit.

Sources: Weather Data folder via weather_evaluation.py (RAWS 10-min mean at 20 ft; gust = prior-hour peak); Week 6 workbook; UQ proposal v0.5 (W5 = Week 5 Platform tab). Assumed values are labelled.