M3 Firefighter Safety: Quantified Uncertainty, Version 1.0

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. First version of the M3 uncertainty calculation; builds on the safety margin in UQ proposal v0.5.

Bottom line

P(M < 0), the chance the fire reaches the crew before they are out, for a 45-minute escape at Camp's first burnovers is 1% with an Alta X if gusts are ignored and 84% if they are not.

Carr: flyable, still at risk. Mule Mountain gusts stayed at or below 21 mph before the 26 July fire whirl, so every listed drone could fly. P(M < 0) for a 45-minute escape is still 21%: about half from a missed detection pass (Pd 0.9), half from relay retries and egress.

Egress time is the missing measurement. Crew positions and escape routes at the burnovers are not in project records, so results are shown for 30, 45 and 60 minutes.

Who uses it

Crew level, lookouts and crew bosses: this is their number; LCES depends on a warning arriving with more time than the escape takes. Division supervisors and the safety officer: they set trigger points from it. Incident commander: decides whether a crew is committed in the conditions.

Drone types serving M3 and their parameters

| Drone | Type | Parameters | Wind limit | M3 role |
| --- | --- | --- | --- | --- |
| FVR-90 | NWCG Type 1 FW | 12 to 18 h; thermal video; flew as airborne relay in TCL 1 | Not found | Overwatch of crews and escape routes; radio relay |
| SuperVolo | VTOL FW research | 8 h; DoodleLabs relay at 1,200 to 1,500 ft MSL | Not found | Alternate relay behind terrain |
| Alta X | NWCG Type 3 rotor | 17 min average sortie; under 2,000 ft AGL | 22 mph (secondary) | Close checks of escape routes and safety zones |
| DJI Matrice 200 v2 (research) | Type 3 / 4 band | Sonic anemometer profiles near the fire front | Not in library | Measures local wind for trigger points |

Live weather during the crew hazards

| Fire | Critical window | Station | Sust. max mph | Gust max mph | Min RH % | Min 10-h fuel moist. % | Wind from |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Camp | 8 Nov 2018, 06:00 to 18:00 PST | JBGC1 Jarbo Gap Raws | 18 | 40 | 11 | 5 | NNE to NE (14 to 42°) |
| Carr | 26 Jul 2018, 12:00 to 23:00 PDT | MMOC1 Shasta - Mule Mountain | 9 | 21 | 7 | 2 | variable, W to NW by evening |
| LNU | 18 Aug 12:00 to 19 Aug 2020 12:00 PDT | ATLC1 Atlas Peak Raws | 29 | 41 | 8 | 4 | SW to WSW (213 to 255°) to 20:00, then WNW |

Camp: in its 07:00 hourly report, before the 07:50 burnovers, Jarbo Gap read 14 mph sustained, 28 mph gusts and 21% humidity; at the Rattlesnake Flats burnover hour (15:00) it read 10 mph, 28 mph gusts, 12%. Carr: at 18:00 on 26 July, before the 18:38 aerial warning, Mule Mountain read 3 mph sustained, 21 mph gusts, 44 °C and 7% humidity: light wind at this station during a wind-driven run with a fire whirl (v0.5).

Crew and fireline tracking. Avenza Maps GPS tracks give each firefighter's position hour by hour; set against the drone fireline they give distance to the edge and real T_egress. Where smoke compromises visibility and escape routes, the drone's thermal view stands in for the lookout and marks no-work zones. Firing operations at the start of a shift need the same gate: at Camp's Rattlesnake Flats a firing operation was planned for 14:15, the wind shifted at 14:45 and the burnover came at 15:15.

How the uncertainty is quantified

Safety margin: M = T_fire - (T_detect + T_notify + T_egress), headline P(M < 0). T_detect: pass every R = 15 min (assumed), flies with wind gate p, sees the hazard with Pd = 0.9 (assumed). T_notify: retries every 2 min (assumed), each delivered with s = 52%, the TCL 1 telemetry track sharing rate with four ground nodes (Wu et al 2025, pp.9-11), used as a stand-in for alert delivery. T_fire: 85 min at Camp (first 911 call 06:25 to first burnovers 07:50) and 62 min at Carr (aerial warning 18:38 to mayday 19:40). Monte Carlo, 20,000 draws per cell.

| P(M < 0) by escape time | 30 min | 45 min | 60 min |
| --- | --- | --- | --- |
| Camp first burnovers, Alta X, sustained | under 1% | 1% | 7% |
| Camp first burnovers, Alta X, gusts | 78% | 84% | 91% |
| Camp first burnovers, Type 1 if 30 mph, gusts | 2% | 7% | 22% |
| Carr 26 July, any drone listed, gusts | 2% | 21% | 100% |

Kinds of uncertainty. p: aleatory (gusts). Pd, R, retry: epistemic, assumed for v1.0. s: measured once in TCL 1. T_egress: epistemic and the largest; it needs crew positions and routes. T_fire is treated as fixed per case; v1.1 samples it from the spread model ensemble.

v1.1 needs: Avenza crew tracks or other crew positions and escape routes at the Camp and Carr burnovers; radio and dispatch logs for real T_notify; a plume-turbulence gate (Pioneer); a measured alert delivery rate (TCL 1 telemetry was over 90% with three ground nodes).

Sources: Weather Data folder via weather_evaluation.py (RAWS 10-min mean at 20 ft; gust = prior-hour peak); Week 6 workbook; UQ proposal v0.5 (W5 = Week 5 Platform tab). Assumed values are labelled.