M4 Water Drop Suppression: Quantified Uncertainty, Version 1.0

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. First version of the M4 uncertainty calculation; mission remains a GAP in the library.

Bottom line

No drone in the project library has dropped water. Every M4 candidate is a test or demonstrator (W5), so M4 v1.0 quantifies what the weather would have allowed, not what was done.

20 to 44 m of drift. A release from 30 m at Camp's first-day winds (sustained 18 mph, gusts 40 mph) moves the load that far downwind before any aiming correction.

Throughput is the other wall. Week 6 records about 35 L/h for a small drone against 10,345 L/h for a large airtanker; swarms of 20 to 100 gal aircraft close part of that only if many fly at once.

Who uses it

Incident commander, operations section chief and air tactical group supervisor: they order and sequence drops and need the chance a drop lands on target when crewed aircraft are grounded. Crews on the line: they work under the drop, so drift and timing are also a safety number for them.

Drone types for M4 and their parameters (W5 unless noted)

| Aircraft | Status | Load | Wind | M4 role |
| --- | --- | --- | --- | --- |
| Rain and Sikorsky autonomous Black Hawk / Firehawk | Demonstrator; CAL FIRE agreement 2025 | 324 gal bucket (demo); 1,000 gal tank class | Flew hands-off in gusts to 30 kt | Direct attack at helicopter scale |
| Seneca Argo-1 swarm (5 drones) | Tested by CAL FIRE, July 2026 | 20 gal water plus foam each | Not found; IR targeting | Initial attack on small fires |
| FireSwarm Thunder Wasp GT | Tested (XPRIZE finalist) | Up to 100 gal | Not found | Initial attack; bulk water |
| Large UAS or optionally piloted aircraft (Joby class) | Concept (ConOps p.19) | Bulk water | Not found | Water drop instead of retardant |

Live weather on the fires

| Fire | Critical window | Station | Sust. max mph | Gust max mph | Min RH % | Min 10-h fuel moist. % | Wind from | Firehawk 30 kt gate, gusts |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Camp | 8 Nov 2018, 06:00 to 18:00 PST | JBGC1 Jarbo Gap Raws | 18 | 40 | 11 | 5 | NNE to NE (14 to 42°) | 92% |
| Pioneer | 29 Aug 2016, 08:00 to 23:00 MDT | BCFI1 Bearskin Creek | 9 | 25 | 16 | n/a | E in the morning, SW to W after 15:00 | 100% |
| LNU | 18 Aug 12:00 to 19 Aug 2020 12:00 PDT | ATLC1 Atlas Peak Raws | 29 | 41 | 8 | 4 | SW to WSW (213 to 255°) to 20:00, then WNW | 84% |
| Carr | 26 Jul 2018, 12:00 to 23:00 PDT | MMOC1 Shasta - Mule Mountain | 9 | 21 | 7 | 2 | variable, W to NW by evening | 100% |

Camp's critical day is the hard case: the Alta X class flies in 1 of 13 hours by gusts, while a 30 kt (34.5 mph) aircraft flies in all but the 06:00 hour. Carr and Pioneer stayed within the 30 kt Firehawk figure; wind limits for the other listed aircraft were not found. Pioneer's run had pyroconvective updrafts, which a surface wind gate does not see.

How the uncertainty is quantified

Effective water on target per hour: V_eff = n x S x V x P(hit) x p, with n aircraft, S sorties per hour = 60 / (2 transit + drop + turnaround), V load per sortie, p the wind gate. P(hit) is the chance the drop centroid lands within the target half-width w: with drift d = u x t_fall, t_fall = sqrt(2h/g) = 2.47 s at h = 30 m (assumed release height), and aiming error σ, P(hit) = Φ((w - d_residual)/σ) - Φ((-w - d_residual)/σ), where d_residual is the drift not corrected by the aim. Wind uncertainty enters twice: through p and through d.

| Fire | Sust. / gust mph | Drift m (h = 30 m) | Gust spread m (uncorrectable) | p, 30 kt gate |
| --- | --- | --- | --- | --- |
| Camp | 18 / 40 | 20 to 44 | 24 | 92% |
| LNU | 29 / 41 | 32 to 45 | 13 | 84% |
| Carr | 9 / 21 | 10 to 23 | 13 | 100% |
| Pioneer | 9 / 25 | 10 to 28 | 18 | 100% |

The gust spread is the part of the drift an aim made on sustained wind cannot remove. This is a no-drag, instant-wind estimate from 20 ft winds; drag and stronger flight-level wind push it in opposite directions, so the sign of the bias is unknown.

v1.1 needs: measured drop accuracy against wind from the Seneca and Firehawk tests; release heights and loads per aircraft; target width by fuel type and fuel loading (LANDFIRE).

Sources: Weather Data folder via weather_evaluation.py (RAWS 10-min mean at 20 ft; gust = prior-hour peak); Week 6 workbook; UQ proposal v0.5 (W5 = Week 5 Platform tab). Assumed values are labelled.