M5 Logistics Support: Quantified Uncertainty, Version 1.0

NASA Aerial Wildfire Project, ACERO Disaster Lab working group. 20 October 2026. First version of the M5 uncertainty calculation; questions as on the Week 6 M5 tab.

Bottom line

7.1 km becomes 4.7 km, then zero. Loaded radius of an Alta X carrying 22 lb (10 kg, about 10 L of drinking water): 7.1 km in still air, 4.7 km into Camp's 18 mph sustained headwind, and nothing into its 40 mph gusts (above the assumed 50 km/h cruise; the airframe top speed is 60 mph, and its 22 mph limit would ground it first).

P(on time) on Camp's first day: 8% to 100% for a 3 km delivery inside a 30-minute window; across the whole Camp perimeter period, gusts ground the Alta X in 32% of hours.

Small drones carry litres, crews need loads. At 3.1 deliveries per hour one Alta X moves about 31 kg/h; heavy-lift aircraft in development (Elroy Chaparral, 500 lb over 300 nm; Piasecki KARGO II, 1,500 lb) are the M5 class.

Who uses it

Crew bosses and division supervisors: whether water, hose, pumps and food arrive when the crew needs them, especially far from roads (Pioneer) or when roads are jammed by evacuation (Camp). Logistics section chief and incident commander: how many aircraft to order and which crews to supply by air.

Drone types serving M5 and their parameters

| Aircraft | Status | Payload and endurance | Wind limit | M5 role |
| --- | --- | --- | --- | --- |
| Alta X (Freefly) | In service, flown with ACERO | 35 lb max; 50 min empty, 35 min at 8 lb, 22 min at 22 lb; 60 mph top speed | 22 mph (secondary) | Light resupply under 10 kg |
| Type 3 / 4 agency small UAS | In service | Light loads; Type 3 range 5 mi, Type 4 under 2 mi | Not published | Radios, batteries, first aid |
| Elroy Air Chaparral | In development (W5) | 500 lb over 300 nm | Not found | Crew resupply from staging |
| Piasecki KARGO II | In development (W5) | 1,500 lb | Not found | Water, hose and pumps in bulk |

Live weather on the fires

| Fire | Critical window | Station | Sust. max mph | Gust max mph | Min RH % | Min 10-h fuel moist. % | Wind from |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Camp | 8 Nov 2018, 06:00 to 18:00 PST | JBGC1 Jarbo Gap Raws | 18 | 40 | 11 | 5 | NNE to NE (14 to 42°) |
| LNU | 18 Aug 12:00 to 19 Aug 2020 12:00 PDT | ATLC1 Atlas Peak Raws | 29 | 41 | 8 | 4 | SW to WSW (213 to 255°) to 20:00, then WNW |
| Pioneer | 29 Aug 2016, 08:00 to 23:00 MDT | BCFI1 Bearskin Creek | 9 | 25 | 16 | n/a | E in the morning, SW to W after 15:00 |
| Carr | 26 Jul 2018, 12:00 to 23:00 PDT | MMOC1 Shasta - Mule Mountain | 9 | 21 | 7 | 2 | variable, W to NW by evening |

Daytime heat also cuts payload: Carr reached 44 °C at Mule Mountain and LNU 38 °C in the afternoon at Atlas Peak, which raises density altitude and shortens loaded endurance (Week 6 M5 Q2 derate).

How the uncertainty is quantified

Probability a requested delivery arrives in its window: P(on time) = p x P(complete | fly) x P(cycle <= window). Loaded radius into a headwind w at airspeed V: r = t_usable / (1/(V - w) + 1/(V + w)), t_usable = endurance loaded - reserve. Cycle = round trip + hold + turnaround (Week 6 M5 Q4). Inputs: endurance 22 min at 22 lb (W5), reserve 5 min, V = 50 km/h, hold 2 min, turnaround 10 min, crew at 3 km (all four assumed). P(complete | fly) = 1 until a delivery success rate is measured.

| Fire, Alta X 10 kg | Radius km at peak sust. / gust | Cycle min at peak sust. / gust | P(on time), share of day-1 hours | p gust, whole fire |
| --- | --- | --- | --- | --- |
| Camp | 4.7 / 0.0 | 23 / none | 8% to 100% | 68% |
| LNU | 0.9 / 0.0 | 68 / none | 60% to 80% | 82% |
| Pioneer | 6.5 / 2.5 | 20 / 32 | 94% to 100% | 98% |
| Carr | 6.5 / 3.8 | 20 / 25 | 100% to 100% | 100% |

Kinds of uncertainty. p and w: aleatory, from station wind. Airspeed, turnaround, success rate: epistemic, assumed for v1.0. Crew demand (Week 6 M5 Q4) is not yet in project records, so fleet size is not computed.

v1.1 needs: crew demand per shift (water, hose, pumps); measured Alta X loaded cruise speed and wind limit; distances from staging to crews on Camp and Pioneer; heavy-lift specifications.

Sources: Weather Data folder via weather_evaluation.py (RAWS 10-min mean at 20 ft; gust = prior-hour peak); Week 6 workbook; UQ proposal v0.5 (W5 = Week 5 Platform tab). Assumed values are labelled.